Master MCU

Node 3 MCU

Node-1 MCU

Node-2 MCU

Back-Up MCU

CAN-Based Multi-Node ESP32 System for Fault Detection and Autonomous Task Reallocation to a Backup Node with IoT Dashboard Monitoring

Overall project datasheet and pin connections

Project: CAN-Based Multi-Node ESP32 System for Fault Detection and Autonomous Task Reallocation to a Backup Node with IoT Dashboard Monitoring.

This is the consolidated hardware connection plan for your full project using 1 Master ESP32 (38-pin), 4 Sensor ESP32 nodes (30-pin), and 5 SN65HVD230 CAN transceiver modules.

1. System architecture

Master ESP32 — 38-pin

Fault detection · OLED · Wi-Fi

GPIO 5 (TX) / GPIO 4 (RX)

SN65HVD230 #1

CAN transceiver

Shared CANH / CANL bus

Node 1

SN65HVD230 #2

DHT11

Node 2

SN65HVD230 #3

Vibration sensor

Node 3

SN65HVD230 #4

MPU6050

Node 4 — Backup

SN65HVD230 #5

DHT11 + vibration + MPU6050

Master Wi-Fi → Supabase → React dashboard

The wiring below uses proposed GPIO assignments for the classic ESP32. Verify the exact board variant and module labels before powering the system.

2. Master ESP32 pin connections

| Master ESP32 pin | Connect to | Function |
| --- | --- | --- |
| 3V3 | SN65HVD230 #1 VCC | Transceiver power |
| GND | Transceiver GND and OLED GND | Ground reference |
| GPIO 5 | Transceiver TXD / D | CAN transmit |
| GPIO 4 | Transceiver RXD / R | CAN receive |
| GPIO 21 | OLED SDA | I²C data |
| GPIO 22 | OLED SCL | I²C clock |
| GPIO 18 | Relay IN1 (optional) | Relay channel 1 |
| GPIO 19 | Relay IN2 (optional) | Relay channel 2 |
| GPIO 2 | LED through resistor (optional) | Status indicator |
| GPIO 23 | Push-button to GND (optional) | Reset/recovery request |

The ESP32's built-in TWAI controller implements CAN protocol handling, while the SN65HVD230 handles the electrical CAN bus signals.

Espressif Documentation

+1

3. All four sensor nodes

Node 1 — DHT11

| DHT11 pin | ESP32 Node 1 |
| --- | --- |
| VCC | 3.3 V, if supported by the module |
| GND | GND |
| DATA | GPIO 16 |

Node 2 — vibration sensor

| Sensor pin | ESP32 Node 2 |
| --- | --- |
| VCC | Module-rated supply |
| GND | GND |
| DO | GPIO 17 |
| AO (if available) | ADC-capable GPIO, optional |

Node 3 — MPU6050

| MPU6050 pin | ESP32 Node 3 |
| --- | --- |
| VCC | Supply supported by the breakout |
| GND | GND |
| SDA | GPIO 21 |
| SCL | GPIO 22 |
| INT (optional) | GPIO 27 |

Node 4 — universal backup node

| Component signal | ESP32 Node 4 |
| --- | --- |
| DHT11 DATA | GPIO 16 |
| Vibration sensor DO | GPIO 17 |
| MPU6050 SDA | GPIO 21 |
| MPU6050 SCL | GPIO 22 |
| MPU6050 INT (optional) | GPIO 27 |
| All sensor grounds | GND |

Node 4 uses its own sensors and local I²C bus. Its MPU6050 connections must not be wired to Node 3's I²C bus.

4. SN65HVD230 module connections — repeat five times

The official TI IC datasheet identifies the following pins.

Texas Instruments

| IC pin | Signal | Connection |
| --- | --- | --- |
| 1 | D / TXD | ESP32 GPIO 5 |
| 2 | GND | ESP32 GND |
| 3 | VCC | ESP32 3V3 |
| 4 | R / RXD | ESP32 GPIO 4 |
| 5 | Vref | Leave unconnected |
| 6 | CANL | Shared CANL |
| 7 | CANH | Shared CANH |
| 8 | RS | GND for high-speed mode |

Repeat this for the Master, Node 1, Node 2, Node 3 and Node 4. If your module has different printed header labels or onboard components, follow its specific module schematic rather than assuming the IC pin numbers match the header order.

5. Complete CAN bus and termination

120 Ω resistor

At first physical end, across CANH–CANL

CANH / CANL twisted-pair trunk

Master

Module #1

Nodes 1–2

Modules #2–3

Nodes 3–4

Modules #4–5

120 Ω resistor

At opposite physical end, across CANH–CANL

- Connect every CANH terminal to the same CANH trunk and every CANL terminal to the same CANL trunk.

- Use only two 120 Ω termination resistors, one at each physical end. Check whether your modules already contain termination resistors.

- With power off and both terminators installed, approximately 60 Ω measured between CANH and CANL is a useful initial check.

- Start testing with two boards before connecting all five.

6. Power supply and common ground

| Item | Connection requirement |
| --- | --- |
| Master and four nodes | Each powered from a suitable regulated supply or USB |
| SN65HVD230 modules | 3.3 V supply |
| Sensor modules | Their individual specified supply voltage |
| CAN bus | CANH and CANL signal wires |
| Ground | Common reference for non-isolated prototype electronics |

Do not apply 12 V directly to ESP32 GPIOs, sensor signal pins, or SN65HVD230 VCC.

1. CAN module pin connections for every ESP32

| SN65HVD230 module pin | Master ESP32 | Node 1 | Node 2 | Node 3 | Node 4 |
| --- | --- | --- | --- | --- | --- |
| VCC | 3V3 | 3V3 | 3V3 | 3V3 | 3V3 |
| GND | GND | GND | GND | GND | GND |
| TXD / D | GPIO 5 | GPIO 5 | GPIO 5 | GPIO 5 | GPIO 5 |
| RXD / R | GPIO 4 | GPIO 4 | GPIO 4 | GPIO 4 | GPIO 4 |
| CANH | Shared CANH | Shared CANH | Shared CANH | Shared CANH | Shared CANH |
| CANL | Shared CANL | Shared CANL | Shared CANL | Shared CANL | Shared CANL |
| RS, if exposed | GND* | GND* | GND* | GND* | GND* |
| Vref, if exposed | Leave open | Leave open | Leave open | Leave open | Leave open |